TECHNICAL DESIGN REPORT

Software Design Description (SDD)

Hardware Design Description (HDD)

Solar-Powered Smart Irrigation System

Weather Integrated, AI-Assisted

CENG318 — Microprocessors

2025-2026 Spring Semester

| Project Name | Solar-Powered Smart Irrigation System (Weather Integrated) |
| --- | --- |
| Document Type | Technical Design Report (SDD + HDD) |
| Version | 2.0 |
| Date | April 28, 2026 |
| Status | Draft — Implementation Phase |
| Instructor | Dr. Muhammet Ünal |

Prepared By

| Name | Student ID | Role |
| --- | --- | --- |
| Şevval Özik | 23118080036 | Mobile/Web UI — UI/UX Dashboard |
| Berke Kocakavak | 23118080059 | Hardware/IoT — Network & IoT |
| Mehmet Köksal (PM) | 23118080060 | Backend — AI Decision Algorithm |
| Salih Akoğlu | 23118080072 | Backend — Database Architecture |
| Zeynep Yamaç | 23118080080 | Hardware/IoT — Embedded Core & Hardware |
| Ediz Gün | 23118080082 | Mobile/Web UI — Frontend Integration |

Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 21.03.2026 | Project Team | Preliminary Design Report (PDR) — initial version |
| 1.1 | 22.03.2026 | Mehmet Köksal | Decision algorithm section updated |
| 1.2 | 24.03.2026 | Berke Kocakavak | Network and IoT sections updated |
| 1.3 | 25.03.2026 | Zeynep Yamaç | Hardware overview updated |
| 1.4 | 26.03.2026 | Şevval Özik | Use cases and interface requirements updated |
| 2.0 | 28.04.2026 | Project Team | Technical Design Report — full architectural redesign of decision engine (LLM + Fuzzy hybrid), detailed module design, web dashboard design, API spec, expanded risk analysis. |

Table of Contents

1. Introduction	8

1.1 Purpose	8

1.2 Scope	8

1.3 Definitions, Acronyms, and Abbreviations	8

1.4 References	9

1.5 Document Overview	9

2. Design Considerations	9

2.1 Design Goals	9

2.2 Architectural Drivers	10

2.3 Assumptions and Dependencies	10

2.4 Constraints	10

3. System Architecture	11

3.1 Architectural Style	11

3.2 High-Level Component Diagram	11

3.3 Operating Modes	12

3.4 Communication Protocols	12

4. Online Decision Engine - Cerebras LLM	13

4.1 Rationale	13

4.2 Three-Stage Decision Pipeline	13

4.3 Provider Selection - Why Cerebras	14

4.4 Cerebras API Integration	14

4.5 Weather Integration – Open Meteo	15

4.6 Prompt Design	15

4.6.1 System Prompt	15

4.6.2 User Prompt	16

4.7 Dynamic Polling	16

4.8 End to End Sequence	16

5. Offline Decision Engine - Fuzzy Logic	17

5.1 Rationale	17

5.2 System Inputs	17

5.3 Membership Functions	18

5.4 Rule Logic & Output	18

5.5 Implementation Strategy	18

5.6 Offline Decision Loop	18

6. Backend Layer Detailed Design	19

6.1	Technology Stack	19

6.2 Module Decomposition	19

6.3 Database Read/Write Pattern Around a Decision	19

6.4 Concurrency Model	20

6.5 Error Handling and Fallback Ladder	20

6.6 Token-Cost Discipline	20

7. Database Layer Detailed Design	21

7.1 Storage Engine	21

7.2 Logical Schema	21

7.3 Key Schema Excerpt — irrigation_log	21

7.4 Retention and Archival	22

7.5 Backups	22

8. Presentation Layer Detailed Design	22

8.1 Technology Stack	22

8.2 Application Routes	22

8.3 Component Tree (key components)	23

8.4 State Management	23

8.5 Authentication and Route Protection	23

8.6 Responsive Design and Mobile	23

8.7 LCD Screen (1602)	24

8.8 UX Notes	24

9. Communication Architecture and API Specification	24

9.1 Endpoint Inventory	24

9.2 Decision Endpoint — Request and Response	25

9.2.1 Request — POST /api/v1/decision	25

9.2.2 Response (200)	25

9.2.3 Response (Fallback / Schema Failure)	25

9.3 ESP32 Authentication	25

9.4 WebSocket Event Stream	26

9.5 Rate Limits	26

10. Use Case Realizations	26

10.1 UC-01 — Automatic Irrigation (online)	26

10.2 UC-02 — Rain Postponement	26

10.3 UC-03 — Manual Control	26

10.4 UC-04 — Report Viewing	27

10.5 UC-05 — System Fault Handling	27

11. Interface Design	27

11.1 User Interfaces	27

11.2 Hardware Interfaces	27

11.3 Software Interfaces	28

11.4 Communication Interfaces	28

12. Hardware Architecture	30

12.1 Overview	30

12.2 Bill of Materials	30

12.3 Hardware Functional Requirements (consolidated from PDR)	30

13. ESP32 Pin Assignment and Sensor Wiring	31

13.1 Pin Map	31

13.2 Sensor Wiring Notes	31

13.3 Relay and Pump Circuit	31

14. Power System Design	32

14.1 Power Architecture	32

14.2 Power Budget	32

14.3 Energy Management Strategy	33

15. Embedded Firmware Design (ESP32)	33

15.1 Toolchain	33

15.2 Module Decomposition	33

15.3 Top-Level State Machine	34

15.4 OTA Updates	34

15.5 Watchdog and Crash Recovery	34

16. Risk Analysis and Mitigation	36

16.1 Risk Register	36

16.2 Detailed Failure Modes - New Findings	36

16.2.1 R-03 - Sensor Poisoning / Hardware Stuck Output	37

16.2.2 R-07 - Decision Coupled to Database Write	37

16.2.3 R-08 - LLM Hallucination of JSON Keys or Types	37

16.2.4 R-09 - Cold Boot Without Weather or Database	37

16.3 Reliability Estimate	38

17. Testing Strategy	38

17.1 Test Levels	38

17.2 Test Matrix — Decision Engine	38

17.3 CI	39

18. Deployment and Maintenance	39

18.1 Deployment Topology	39

18.2 Configuration	39

18.3 Maintainability Guidelines	39

19. Project Timeline	39

20. Team Structure and Responsibilities	40

PART I

SOFTWARE DESIGN DESCRIPTION (SDD)

# 1. Introduction

## 1.1 Purpose

This Technical Design Report (TDR) defines the detailed software and hardware design of the Solar-Powered Smart Irrigation System. It builds on the Preliminary Design Report (PDR v1.4, 26.03.2026), which captured the requirements (SRS) and high-level hardware specification (HRD), and translates those requirements into concrete architectural decisions, module-level designs, interface specifications, data structures, and risk mitigations that the team will implement during the remaining six weeks of the project.

The most significant architectural change since the PDR is the redesign of the AI decision engine. The original ML-based approach has been replaced with a hybrid two-mode architecture: an online mode driven by a Cerebras-hosted Large Language Model (llama3.1-8b) and an offline mode running pure Fuzzy Logic on the ESP32. This decision is documented in detail in Section 4 and Section 5 of this report.

## 1.2 Scope

The scope of this design covers the full end-to-end system: the ESP32-based field layer, a C# (.NET 8/9) backend layer that manages the decision logic and external API integrations, a PostgreSQL database for persistent state and historical reporting, and a React-based web dashboard. The mobile experience is delivered by the same web application using a responsive layout. The design assumes deployment on a single residential site in Ankara, Turkey, with one ESP32 device per installation.

## 1.3 Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| ESP32 | Wi-Fi and Bluetooth enabled microcontroller manufactured by Espressif Systems. |
| SDD | Software Design Description (IEEE 1016-aligned). |
| HDD | Hardware Design Description. |
| SRS | Software Requirements Specification (delivered in PDR v1.x). |
| LLM | Large Language Model. In this project, llama3.1-8b served via Cerebras Cloud. |
| Cerebras | Cloud LLM provider used as the online decision backbone (≈1800 tokens/sec). |
| Fuzzy Logic | Rule-based decision technique using partial set membership; used in offline mode. |
| Open-Meteo | Free, no-key weather forecast API used as the weather data source. |
| MQTT | Lightweight publish/subscribe messaging protocol for IoT. |
| REST | Representational State Transfer; used between dashboard, backend, and ESP32. |
| JWT | JSON Web Token; used for authentication between dashboard and backend. |
| I2C | Inter-Integrated Circuit serial bus (used for the 1602 LCD). |
| ADC | Analog-to-Digital Converter (ESP32 pin used for moisture sensor and battery). |
| Deep Sleep | ESP32 ultra-low-power mode (~10 µA) used between irrigation checks. |
| UC | Use Case. |
| FR / NFR | Functional / Non-Functional Requirement. |
| DXA | Twentieths of a point — internal docx unit (1440 DXA = 1 inch). |

## 1.4 References

- IEEE Std 1016-2009 — Standard for Information Technology — Systems Design — Software Design Descriptions.

- IEEE Std 830-1998 — Recommended Practice for Software Requirements Specifications.

- ESP32 Technical Reference Manual (Espressif Systems).

- Cerebras Inference API Documentation — https://inference-docs.cerebras.ai

- Open-Meteo Forecast API — https://open-meteo.com/en/docs

- Preliminary Design Report v1.4 — 26.03.2026 (this team).

## 1.5 Document Overview

Part I (Sections 1–11) covers software design: introduction, design considerations, system architecture, the online and offline decision engines, backend, database, presentation, communication, use case realizations, and interface design. Part II (Sections 12–15) covers hardware design: hardware architecture, pin assignment, power system, and embedded firmware. Part III (Sections 16–20) covers risk analysis, testing strategy, deployment, project timeline, and team responsibilities.

# 2. Design Considerations

## 2.1 Design Goals

The design optimizes for five concrete goals, listed in priority order. When trade-offs arise, higher-numbered goals are sacrificed first.

- Plant safety: irrigation must continue even when the cloud, the LLM, or the local Wi-Fi is unavailable. Loss of data is acceptable; failure to water is not.

- Energy autonomy: the system must run indefinitely on a single 6 V solar panel and 18650 cell, with no grid connection.

- Decision quality: when network is available, the system uses contextual reasoning (weather forecast, time of day, plant species, last irrigation time) rather than fixed thresholds.

- Cost: bill-of-materials kept under student-budget level; all cloud services chosen are free-tier (Cerebras free tier, Open-Meteo, free PostgreSQL hosting).

- Maintainability: every module is independently deployable; the firmware, backend, database, and frontend can each be updated without touching the others.

## 2.2 Architectural Drivers

The following drivers shaped the most consequential design decisions in this report:

| Driver | Implication on Design |
| --- | --- |
| Intermittent Wi-Fi | Mandatory dual-mode decision engine (online + offline). See Sections 4 and 5. |
| ESP32 RAM (520 KB) | HTTPS, JSON parsing, weather fetch and prompt construction are pushed off-device into the C# backend. ESP32 only does sensors, sleep, pump, and a local fallback Fuzzy controller. |
| Solar energy budget | Dynamic polling — the LLM decides the next wake-up time per cycle (between 10 and 240 minutes). See Section 4.7. |
| Plant-aware reasoning | The backend takes a free-text plant name from the user (e.g. "tomato") and feeds it directly to the LLM, exploiting the model's prior knowledge of crop water needs. No category enumeration is required. |
| Token cost discipline | A 3-stage decision pipeline (Pre-Rules → LLM → Post-Rules) avoids API calls in obvious cases and caps maximum daily token spend. |
| Sensor failure tolerance | A 48-hour timeout overrides the rain sensor pre-rule to defeat stuck-high failure modes (see Section 16). |

## 2.3 Assumptions and Dependencies

- Wi-Fi internet access is intermittently available at the installation site. The system tolerates outages of any duration.

- The 6 V solar panel receives at least 4 hours of direct sunlight per day on average.

- The Open-Meteo public API remains free and accessible at api.open-meteo.com.

- The Cerebras Cloud free tier (1 M tokens/day) remains available; if revoked, the system falls back automatically to offline Fuzzy mode.

- Users have access to a modern web browser (Chrome ≥ 110, Firefox ≥ 110, Safari ≥ 16) on a smartphone or desktop.

- A water source (tank or tap) is located near the pump, and the pump head is below water level (submersible pump assumption).

## 2.4 Constraints

- Energy budget — average draw must remain below the daily harvest of the solar panel (≈ 6 V × 1 W panel ≈ 80 mAh/day usable into 18650 at 70 % efficiency, project-budget assumption).

- ESP32 — 520 KB SRAM, 4 MB Flash, no hardware floating-point accelerator beyond the FPU; no native HTTPS+TLS streaming JSON parser.

- Cerebras free tier — 1 M tokens/day, ≈ 30 RPM rate limit. The design budgets ≤ 300 tokens per decision and ≤ 1 decision per 10 minutes per device, well below the cap.

- Pump protection — maximum continuous on-time of 30 seconds enforced in firmware and re-enforced as a Post-Rule in the backend.

- Privacy — all communication encrypted with TLS; no personally identifying information beyond the username is stored.

# 3. System Architecture

## 3.1 Architectural Style

The system follows a four-layer architecture, partitioned by physical location and trust boundary. Each layer communicates only with its direct neighbours, and crossings are formalised as REST or MQTT interfaces. The layers, top-down by user proximity, are:

- Presentation Layer — React (Vite) web dashboard, also rendered responsively as a mobile web view. Optional 1602 LCD on the device for at-a-glance status.

- Application Layer — C# (.NET 8/9) backend exposing a REST API. Hosts the decision pipeline, the Cerebras and Open-Meteo integrations, and the authentication service.

- Data Layer — PostgreSQL relational database holding sensor history, irrigation log, user accounts, and configuration.

- Field Layer — ESP32 firmware reading sensors, driving the relay/pump, executing the offline Fuzzy controller, and managing deep sleep.

## 3.2 High-Level Component Diagram

The end-to-end flow, in a single picture and in plain words:

┌─────────────────────────────────────────────────────────────────┐

│                         PRESENTATION LAYER                      │

│   ┌──────────────────────────┐    ┌──────────────────────────┐  │

│   │   Web Dashboard (React)  │    │  1602 LCD (on device)    │  │

│   │  • Live sensor cards     │    │   • Moisture, pump,      │  │

│   │  • Manual control        │    │     Wi-Fi indicator      │  │

│   │  • Reports & history     │    └────────────┬─────────────┘  │

│   └─────────────┬────────────┘                 │                │

└─────────────────│───────────────────────────────│────────────────┘

│ HTTPS / WebSocket             │ I2C

┌─────────────────▼───────────────────────────────│────────────────┐

│                         APPLICATION LAYER       │                │

│  ┌──────────────────────────────────────────┐  │                │

│  │      C# / ASP.NET Core Backend (Cloud)    │  │                │

│  │┌────────────────────────────────────┐  │  │                │

│  ││  Decision Pipeline (3 stages)      │  │  │                │

│  ││   1. Pre-Rules                     │  │  │                │

│  ││   2. Cerebras LLM (llama3.1-8b)    │◄─┼──┼─Open-Meteo API │

│  ││   3. Post-Rules / Schema Guard     │  │  │   (forecast)     │

│  │└────────────────────────────────────┘  │  │                │

│  │┌────────────────────────────────────┐  │  │                │

│  ││  Auth (JWT) • REST API • Reports   │  │  │                │

│  │└────────────────────────────────────┘  │  │                │

│  └──────────────┬───────────────────┬───────┘  │                │

└──────────────────│───────────────────│──────────│────────────────┘

│ SQL               │ HTTPS    │

┌──────────────────▼─────┐    ┌────────▼──────────▼────────────────┐

│      DATA LAYER        │    │          FIELD LAYER               │

│   PostgreSQL           │    │   ESP32 (firmware)                 │

│   • sensor_log         │    │   • Moisture, rain, battery sense  │

│   • irrigation_log     │    │   • Pump relay (5 V)               │

│   • users / devices    │    │   • Offline Fuzzy controller (C++) │

│   • config             │    │   • Deep-sleep scheduler           │

└────────────────────────┘    └────────────────────────────────────┘

## 3.3 Operating Modes

The decision engine has two distinct, fully self-contained modes. Mode selection happens on each wake-up, before any sensor reading is sent off-device.

| Mode | Trigger | Decision Path | Latency | External Calls |
| --- | --- | --- | --- | --- |
| Online | Wi-Fi associated and DNS resolves backend | ESP32 → Backend → Pre-Rules → Cerebras LLM → Post-Rules → ESP32 | ≈ 0.7–1.5 s | Cerebras + Open-Meteo + Postgres |
| Offline | Wi-Fi association fails, or backend HTTP timeout | ESP32 local Fuzzy Logic (C++) → Pump | ≈ 50 ms | None |

Once an online decision has been issued, its next_check_min field also dictates how long the ESP32 sleeps before the next mode probe. Offline mode uses a fixed 30-minute fallback interval, which is acceptable because offline operation is by definition a degraded state whose only goal is plant survival.

## 3.4 Communication Protocols

- ESP32 ↔ Backend — HTTPS REST (POST /api/v1/decision). Justification: low frequency (≤ 1 req / 10 min), need for synchronous response, easier authentication.

- Dashboard ↔ Backend — HTTPS REST + WebSocket. WebSocket is used for the live sensor cards on the dashboard so that an irrigation event can push to the user within ~1 s.

- Backend ↔ Cerebras — HTTPS, OpenAI-compatible endpoint, JSON-mode response.

- Backend ↔ Open-Meteo — HTTPS GET, no authentication required.

- Backend ↔ PostgreSQL — TCP, parameterised SQL via psycopg2, statement timeout 2 s.

- ESP32 ↔ Sensors — analog (moisture, battery), digital (rain), I2C (LCD).

# 4. Online Decision Engine - Cerebras LLM

## 4.1 Rationale

In the initial requirements phase (SRS) I planned to train a custom Machine Learning (ML) model for irrigation decisions. But during the design phase, I realized this approach did not fit our strict memory and power limits. I concluded that a standard ML model was not the right choice for this project.

As a result, I switched to a hybrid system (a cloud-based LLM for online mode and on-device Fuzzy Logic for offline mode) for the following reasons:

- Hardware Limits & Reliability: The ESP32 has limited memory. Running an ML model locally when the Wi-Fi drops could cause memory crashes and sudden power drains. For the offline fallback we need a simple, predictable and lightweight algorithm (Fuzzy Logic) to keep the plant alive without overloading the microcontroller.

- Power Saving & Smart Waking: To run entirely on solar power the ESP32 must maximize its deep sleep time. The Cerebras LLM  responds incredibly fast (around 250 ms) allowing the device to wake up, get a decision and go back to sleep in under a second. Also the LLM can calculate exactly when the device should wake up next (next_check_min) based on the weather forecast. This prevents unnecessary wake ups and saves much more battery than a standard ML model.

- Flexibility Without Retraining: A standard ML model would need new data and retraining every time we add a new plant type. An LLM, already has general knowledge about plants. We can simply input a plant name (like "tomato" or "lavender") and it will instantly know its watering needs making the system much easier to expand.

- Avoiding Redundant Data Labeling: Training a custom ML model requires a large and labeled dataset. Even if we fetched a weather dataset for Ankara we still have to manually label each data point as "irrigate" or "do not irrigate." To create these labels we would end up using hardcoded rules anyway. Consequently, the ML model would just learn to mimic those exact same static rules adding unnecessary complexity to the project without actually making the system any smarter.

## 4.2 Three-Stage Decision Pipeline

The decision flows through three stages on every wake up. Stages 1 and 3 are executed entirely on device using C++ and free. Only stage 2 requires a network request and consumes API tokens.

| Stage | Purpose | Effect |
| --- | --- | --- |
| 1) Pre-Rules | Cheap, obvious shortcuts before the LLM is consulted. | If the rain sensor is active OR soil moisture > 80 %, return SKIP without calling the LLM. ≈ 20–30 % of cycles end here saving tokens and latency. |
| 2) LLM Inference | Full contextual decision via Cerebras llama3.1-8b in JSON mode. | Returns a structured action object: {action, duration_sec, next_check_min, reason}. |
| 3) Post-Rules | Safety filter on the LLM output. | duration_sec is clamped to [0, 30], next_check_min to [10, 240], and the action is whitelisted to {IRRIGATE, SKIP}. The JSON schema is parsed and strictly validated using System.Text.Json and FluentValidation. On any deserialization failure or rule violation the system falls back to a safe default (e.g. SKIP, recheck in 30 min) to prevent sending malformed commands to the ESP32. |

## 4.3 Provider Selection - Why Cerebras

I compared three different free tier providers for running the LLM. In the end, I chose Cerebras because of its high speed and generous free tier limits. These two factors are critical because they directly help the ESP32 save battery life:

| Property | Cerebras | Groq | Google Gemini Flash |
| --- | --- | --- | --- |
| Throughput | ~1800 tokens/s | ~750 tokens/s | Medium |
| Free tier | 1 M tokens/day | 14 400 req/day | 1000 req/day |
| Credit card | Not required | Not required | Not required |
| API style | OpenAI-compatible | OpenAI-compatible | Proprietary |
| Latency p50 | ~250 ms | ~600 ms | ~900 ms |

At about 250 ms response time, the ESP32 can stay awake during the request and go back to deep sleep in less than one second for each cycle. We use the llama3.1-8b model because this task is simple and structured so a bigger model is not needed. The smaller model is faster and costs less.

## 4.4 Cerebras API Integration

The system connects to the Cerebras engine using a standard REST API over a secure connection (HTTPS). The integration follows these simple steps:

- Security: We use HTTPS (TLS 1.2) to make sure that the API key and sensor data are encrypted and safe while being sent over the internet.

- Data Format: All data sent to and received from the API uses the JSON format. We follow the OpenAI compatible structure which makes the system easy to update or change in the future.

- Authentication: The system uses a Bearer Token (API Key) to access the service. This key is stored securely on the device’s memory instead of being hardcoded directly into the main script.

- Error Handling: I set a 5 second timeout for the API calls. If the internet is too slow or the API returns an error, the system doesn't get stuck it immediately switches to the Fuzzy Logic Fallback to decide whether to water the plants.

## 4.5 Weather Integration – Open Meteo

I chose Open Meteo for weather data because it is free, does not require an API key and has no rate limits. This makes it very reliable for a student project. The integration works as follows:

- Data Retrieval: During each decision cycle the system sends an HTTPS GET request configured with Ankara's coordinates (39.9334 N, 32.8597 E)

- Key Parameters: Instead of downloading the whole forecast the system only requests data for the next three hours. It specifically looks at three factors: precipitation probability, temperature (at 2m) and relative humidity.

- Failure Handling: If the Open Meteo API fails or the network times out it is not a "fatal" error for the system. In this case, the backend simply informs the LLM that "Weather forecast is unavailable."

- Fallback Strategy: When the weather data is missing the LLM is instructed to take a cautious approach. It will usually perform a small lifeline watering and set a shorter wait time (next_check_min) to try and get the forecast again as soon as possible.

## 4.6 Prompt Design

### 4.6.1 System Prompt

The System Prompt acts as the foundation of the AI’s behavior and includes all the information that remains constant. It defines the controller's role, the local climate of Ankara, and the hardware's physical limits.

- Language Choice: The prompt is written in English. This is a strategic choice because Turkish tokens are 30–50% more expensive due to the way Llama's tokenizer works. To keep the dashboard user friendly the "reason" field provided by the AI is translated into Turkish at the application layer before being displayed.

- Location and Climate: The AI is informed that it is operating in Ankara which has a semi arid climate. It is instructed to be particularly careful about rain forecasts especially during the sudden storms common in spring.

- Hardware Protection: To protect the pump, the prompt sets a maximum irrigation limit of 30 seconds.

- Output Requirements: Instead of a conversational reply the AI is strictly instructed to provide a structured response. This response must include the decision (IRRIGATE or SKIP), the irrigation duration (0–30 seconds), the time until the next check (10–240 minutes) and a brief explanation of the logic used.

### 4.6.2 User Prompt

The User Prompt is generated dynamically during every wake-up cycle and provides the real time data needed for a decision. It includes the following information:

- Real-time Sensor Data: Current soil moisture percentage and the status of the rain sensor (whether it is currently raining or dry).

- Short-term Weather Forecast: The probability of rain, temperature and relative humidity for the next three hours in Ankara.

- Timing Context: The current time and how many hours have passed since the last irrigation cycle.

- Plant Species: The specific plant name is sent as free text. By providing the exact name we allow the LLM to use its own extensive botanical knowledge regarding root depth and water needs. This is much more effective than using simple categories like "high/low water need" as it allows for a more personalized irrigation strategy for each species.

## 4.7 Dynamic Polling

Conventional irrigation controllers poll on a fixed timer like every 15 min. This wastes both energy and tokens. Instead the LLM returns a next_check_min in [10, 240] which the ESP32 uses as its deep sleep duration. Sample policies the model has been observed to follow:

| Scenario | Decision | Rationale |
| --- | --- | --- |
| Dry soil, rain in 1 hour | SKIP, next=60 min | Wait and verify whether rain materialises. |
| Very wet soil | SKIP, next=240 min | Saves 4 hours of polling and ~16 token-burning calls. |
| Just irrigated | IRRIGATE, next=120 min | Allow soil to absorb before re-checking. |
| Dry soil, clear sky | IRRIGATE, next=90 min | Standard re-check after irrigation. |

## 4.8 End to End Sequence

A single operational cycle follows a structured sequence that moves from local sensing to cloud based intelligence and finally to physical action. The process is summarized below:

Initialization and Sensing: The cycle begins when the ESP32 wakes up from its scheduled deep sleep. It immediately gathers data from the local environment, reading the soil moisture percentage, checking the rain sensor for any active precipitation and measuring the battery voltage to ensure the system has enough energy to operate the pump.

Data Transmission and Context Gathering: After collecting the local data the ESP32 sends this information to the backend server. The backend then enriches this data by retrieving the last irrigation time from the PostgreSQL database and fetching the current 3hour weather forecast (probability of rain, temperature and humidity) from the Open Meteo API.

Decision Stages: The system first applies the Pre Rules if it is already raining or the soil is already very wet, the cycle ends immediately to save power and API tokens.

If a decision is needed, the backend builds a user prompt and sends it to the Cerebras LLM. The model analyzes the sensors, weather and plant type to return a specific action (Irrigate or Skip), the duration of the watering and the reasoning behind it.

Finally, the Post Rules stage validates this output ensuring the watering duration and the next check interval are within safe hardware limits.

Execution and Logging: Once the decision is finalized the backend logs the entire transaction including the reason and duration into the database for future reference. It then sends the final instructions back to the ESP32.

Physical Action and Deep Sleep: The ESP32 receives the instructions and if the decision is to irrigate, it activates the relay to run the pump for the specified number of seconds. After the task is complete, the device turns off the pump and enters deep sleep for the duration recommended by the LLM, effectively conserving battery until the next cycle.

# 5. Offline Decision Engine - Fuzzy Logic

## 5.1 Rationale

When the Wi-Fi connection is lost, the ESP32 must still keep the plants alive without relying on the cloud, weather APIs or the backend server. To handle this, a Fuzzy Logic engine runs entirely on the device (using C++). Fuzzy Logic was chosen over a standard if/else rule system because it provides smoother and more realistic decision outputs (e.g., classifying a 39% moisture reading as "partially dry and partially medium") rather than relying on strict, hardcoded thresholds.

## 5.2 System Inputs

Since the offline mode cannot access weather forecasts, it relies strictly on real-time, on-device hardware readings:

Soil Moisture (%): Read via the capacitive sensor on GPIO 34 (ADC) and used as the primary fuzzy variable.

Time of Day: Read from the ESP32’s internal RTC (Real-Time Clock) and used as a fuzzy variable to optimize watering times.

Rain Sensor (Binary): Read via GPIO 35. This acts as a strict "Pre-rule" bypass; if physical rain is detected, the fuzzy calculation is skipped, and irrigation is canceled.

Battery Voltage: Read via GPIO 32. This acts as a safety override to suspend operations if the battery drops below critical levels (3.3V).

## 5.3 Membership Functions

The inputs are mapped into categories to create logical rules:

Soil Moisture: Dry: 0–40% (Starts tapering down after 20%)

- Medium: 20–60% (Peaks strictly at 40%)

- Wet: 40–80% (Considered fully wet past 80%)

Time of Day (Adjusted for Ankara Climate):

- Morning (05:00–10:00): Best time (low evaporation).

- Midday (10:00–17:00): Avoid if possible (high evaporation).

- Evening (17:00–21:00): Good alternative (temperatures dropping).

- Night (21:00–05:00): Low priority.

## 5.4 Rule Logic & Output

The system evaluates the moisture and time variables to determine the necessary pump running time (duration_sec).

If the soil is Dry, the system will irrigate regardless of the time, but durations are shorter during midday (3–5 seconds) to prevent immediate evaporation, and longer in the morning/evening (8–12 seconds).

If the soil is Medium, it only applies short "maintenance" watering (3–5 seconds) during optimal times (Morning/Evening) and skips midday entirely.

If the soil is Wet, irrigation is skipped across all time slots.

## 5.5 Implementation Strategy

Because the ESP32 does not support a C# runtime environment, standard C# fuzzy logic libraries cannot be used on the device. Therefore, the fuzzy membership functions are implemented directly in C++ using lightweight linear interpolation. The output is computed using the centroid (weighted average) defuzzification method. This approach is highly efficient in terms of memory and processing requirements, making it well-suited for resource-constrained embedded hardware.

## 5.6 Offline Decision Loop

1.The ESP32 wakes from deep sleep.

2.Pre Rules: It checks the rain sensor and battery. If it is raining or the battery is low, it skips the cycle and sleeps for 30 minutes.

3.It reads the soil moisture. If it is above 80%, it skips and sleeps for 60 minutes.

4.It reads the current time slot from the RTC.

5.The C++ engine computes the membership grades (e.g. Dry + Morning) and defuzzifies the result to calculate duration_sec.

6.If the duration is 1 second or more, the ESP32 activates the relay to run the pump.

7.Finally, the system enters deep sleep for a fixed 30 minutes before attempting to reconnect to the Wi-Fi. (Dynamic polling via next_check_min is unavailable offline so a fixed 30-minute fallback is used to prioritize plant survival over efficiency).

# 6. Backend Layer Detailed Design

## Technology Stack

The backend decision engine is developed using the C# programming language on the .NET 8/9 platform, providing a high-performance and scalable environment.

- Web Framework: We use ASP.NET Core Web API to handle incoming requests. For security and authentication, IdentityServer or JWT (JSON Web Tokens) are implemented to secure the communication between the ESP32 and the server.

- Validation & LLM Client: Request validation is handled using FluentValidation. The LLM integration is performed via the Semantic Kernel or a custom HttpClient wrapper, targeting the OpenAI compliant Cerebras API endpoint.

- Database & Persistence: PostgreSQL is the primary database, managed through Entity Framework Core (EF Core). While EF Core handles the dashboard logic, raw Dapper queries may be used for high speed decision logging.

- Deployment: The application is hosted as a self contained service on a Linux based VPS, utilizing Kestrel as the primary web server behind an Nginx reverse proxy for SSL/TLS termination.

## 6.2 Module Decomposition

The system follows a modular Clean Architecture approach to ensure maintainability:

- Controllers: DecisionController handles the /api/v1/decision requests from the ESP32 while DashboardController serves the real time sensor data and historical reports to the UI.

- Decision Service: This is the core logic layer that manages the three-stage pipeline (Pre-Rules, LLM Inference, Post-Rules). It is isolated from external dependencies for easier unit testing.

- Weather Service: A dedicated service for the Open Meteo API, featuring a transient fault handling policy for retries and timeouts.

- LLM Integration Service: Manages the communication with Cerebras including prompt construction and JSON response parsing using System.Text.Json.

- Repository Layer: Encapsulates all PostgreSQL operations ensuring that database logic is separated from the business rules.

## 6.3 Database Read/Write Pattern Around a Decision

Every irrigation decision cycle involves two primary database operations to ensure data integrity and context awareness:

Context Fetch (Read): Before the decision is made, the system queries the irrigation_history table via the pg (PostgreSQL) driver in Node.js to find the most recent watering event. This data is used to calculate the time elapsed since the last irrigation, which is then sent to the LLM to prevent overwatering.

Decision Logging (Write): Once the decision is finalized, a new record is created in the irrigation_history table containing the duration (duration_minutes), trigger type, and moisture levels before and after the process.

Reliability Note: To prioritize plant survival, the decision response is returned to the ESP32 even if the database logging fails. Any failed write operations are caught via a try/catch block and logged locally to the server console for manual review.

## 6.4 Concurrency Model

Given the expected load of approximately one request per minute per device, the asynchronous (async/await) nature of ASP.NET Core allows the server to handle multiple requests with minimal thread usage. The weather fetch and the LLM call are performed sequentially because the AI prompt requires the weather data as an input. Despite, the total response time is kept under 1 second, ensuring the ESP32 does not stay awake longer than necessary.

## 6.5 Error Handling and Fallback Ladder

The system uses a Graceful Degradation strategy to handle failures:

- LLM API Issues: If the Cerebras API is down or returns invalid JSON, the system attempts a retry. If it still fails, it defaults to a SKIP decision and schedules a retry in 15 minutes.

- Weather API Issues: If weather data cannot be fetched, the system continues the cycle but marks the forecast as Unavailable in the prompt forcing the LLM to use a safer, conservative watering strategy.

- Database Issues: If the database is unreachable, the decision-making process continues using default values (e.g. assuming 12 hours since last watering) to ensure the system never stops functioning.

## 6.6 Token-Cost Discipline

To stay well within the Cerebras free tier limits (1M tokens/day), the C# backend implements three efficiency rules:

- Pre-Rule Filtering: The system skips the LLM call entirely if soil moisture is already high or if the rain sensor is active, reducing API costs by 20-30%.

- English Prompting: All AI instructions are in English to optimize token usage as Llama tokenizers are more efficient with English text than Turkish.

- Smart Polling: The system utilizes the next_check_min value returned by the AI to put the device into longer deep-sleep cycles when the soil is healthy, significantly reducing the total number of API calls per day.

# 7. Database Layer Detailed Design

## 7.1 Storage Engine

## PostgreSQL was selected as the system of record. The data is structured and time-stamped; a relational engine gives strong indexing for time-range queries (e.g., the Reports endpoint fetching daily/weekly/monthly stats). The Node.js backend connects to it via the pg connection pool.

## 7.2 Logical Schema

| Table | Purpose | Primary key | Notable indexes |
| --- | --- | --- | --- |
| users | Account credentials and profile. | id (INTEGER) | unique(username), unique(email) |
| plants | Plant species per device. | id (UUID) | foreign key to devices |
| sensor_logs | Time-series of every sensor packet posted (soil moisture, temperature, rain probability, battery level) | id (INTEGER) | btree(device_id, ts DESC) |
| irrigation_history | One row per irrigation event, logging duration and consumed water. | id (INTEGER) | btree(device_id, ts DESC) |
| notifications | User-facing alerts and manual pump trigger logs. | id (INTEGER) | btree(user_id, ts DESC) |
| system_settings | Dynamic system configuration like moisture thresholds, max pump runtime, and deep sleep interval. | id (INTEGER) | btree(device_id, ts DESC) |

## 7.3 Key Schema Excerpt — irrigation_history

CREATE TABLE irrigation_history (

id                  SERIAL PRIMARY KEY,

start_time          TIMESTAMP WITHOUT TIME ZONE,

duration_minutes    DOUBLE PRECISION,

trigger_type        VARCHAR(50),

moisture_before     DOUBLE PRECISION,

moisture_after      DOUBLE PRECISION,

liters_consumed     DOUBLE PRECISION

);

## 7.4 Retention and Archival

- sensor_log — kept for historical environment tracking.

- irrigation_history — kept indefinitely to allow the /api/reports endpoint to calculate weekly and monthly water consumption.

## 7.5 Backups

# Routine logical backups can be configured via pg_dump to off-site storage. The cloud hosting provider offers point-in-time recovery to prevent data loss.

# 8. Presentation Layer Detailed Design

## 8.1 Technology Stack

- React 19 (functional components, hooks).

- Vite (dev server, build).

- Tailwind CSS 4 (utility-first styling, responsive breakpoints).

- Recharts 3 (line and bar charts).

- React Router 7 (client-side routing).

- Lucide React (icon set).

## 8.2 Application Routes

| Path | Page | Auth | Purpose |
| --- | --- | --- | --- |
| /login | LoginPage | No | Username/password login. Demo creds: admin / admin123. |
| / | DashboardPage | Yes | Live overview: moisture chart, AI decision card, weather, pump, rain, battery, Wi-Fi cards. |
| /control | ControlPage | Yes | Manual pump on/off with session timer; manual mode temporarily overrides the AI. |
| /reports | ReportsPage | Yes | Daily / weekly / monthly water consumption charts and summaries (UC-04). |
| /history | HistoryPage | Yes | Filterable, sortable table of past irrigation events. |
| /notifications | NotificationsPage | Yes | In-app notification feed; mark-as-read, severity filter. |
| /settings | SettingsPage | Yes | Plant type selection, timing configuration, and alert preferences. |
| /system | SystemStatusPage | Yes | System health monitoring, mode indicator (ONLINE/OFFLINE), fault log. |

## 8.3 Component Tree (key components)

src/

├── App.jsx                        (routes)

├── context/

│   ├── AuthContext.jsx           (login state + token)

│   └── SystemContext.jsx         (live sensor + decision state)

├── hooks/

│   └── useSimulatedData.js       (mock data feed; replaced by WebSocket in integration)

├── components/

│   ├── layout/    Layout, Sidebar, Navbar

│   ├── common/    Card, StatusBadge

│   ├── dashboard/ MoistureCard, MoistureChart, AiDecisionCard,

│   │              WeatherCard, PumpStatusCard, RainStatusCard,

│   │              BatteryCard, WifiStatusCard

│   ├── controls/  manual pump UI

│   ├── reports/   ConsumptionChart, ReportSummary

│   └── notifications/ NotificationBell

└── pages/  (one per route, listed above)

## 8.4 State Management

Two React Contexts are used. AuthContext holds the JWT, the current user, and exposes login() / logout() / isAuthenticated. SystemContext holds the live sensor snapshot and the latest AI decision; it subscribes to a WebSocket from the backend (currently mocked by useSimulatedData for development). All other state is local component state — no Redux, no Zustand, intentionally.

## 8.5 Authentication and Route Protection

A ProtectedRoute wrapper component checks AuthContext.isAuthenticated and redirects to /login when false. The JWT is stored in memory (not localStorage) to mitigate XSS leakage. A refresh token in an HttpOnly, Secure cookie covers re-authentication after page reload.

## 8.6 Responsive Design and Mobile

The dashboard is responsive by Tailwind utility classes; below the md breakpoint (768 px) the sidebar collapses into a hamburger and the dashboard cards reflow into a single column. The same React build is used as the "mobile app" for this project; a native wrapper (PWA install prompt or a thin Capacitor shell) is a stretch goal not on the critical path.

## 8.7 LCD Screen (1602)

The on-device 1602 LCD is treated as a separate, simpler view. The ESP32 firmware writes two lines: line 1 shows soil moisture and pump state; line 2 shows Wi-Fi status and the last decision summary. The LCD is non-interactive; it exists for in-the-garden glance-ability when the user does not have their phone.

## 8.8 UX Notes

- Severity colour code is consistent across the app — info (blue), success (green), warning (amber), danger (red). Defined as Tailwind tokens.

- All actions that consume water (e.g. "Pump On" in /control) require a confirm step.

- Time stamps are shown in user-local time; the underlying records are stored as UTC.

- When the system enters offline mode, a banner is shown across the dashboard so the user is never confused about why the AI reasons sound terser.

# 9. Communication Architecture and API Specification

## 9.1 Endpoint Inventory

| Method | Path | Caller | Auth | Purpose |
| --- | --- | --- | --- | --- |
| POST | /api/v1/auth/login | Dashboard | Public | Credentials → JWT + refresh cookie. |
| POST | /api/v1/auth/refresh | Dashboard | Refresh cookie | Rotate JWT. |
| POST | /api/v1/decision | ESP32 | Device token | Sensor packet → action JSON. Hot path. |
| POST | /api/v1/control/manual | Dashboard | JWT | Manual pump on/off override. |
| GET | /api/v1/sensors/latest | Dashboard | JWT | Latest sensor snapshot. |
| GET | /api/v1/reports?range=daily | Dashboard | JWT | Aggregated water consumption. |
| GET | /api/v1/history | Dashboard | JWT | Paginated irrigation history. |
| GET | /api/v1/notifications | Dashboard | JWT | In-app notifications. |
| PATCH | /api/v1/settings | Dashboard | JWT | Update thresholds and plant name. |
| WS | /ws/live | Dashboard | JWT | Live push of sensor and decision events. |

## 9.2 Decision Endpoint — Request and Response

### 9.2.1 Request — POST /api/v1/decision

POST /api/v1/decision  HTTP/1.1

Authorization: Device <preshared-token>

Content-Type: application/json

{

"device_id": "esp32-001",

"ts":        "2026-04-28T15:32:11+03:00",

"moisture":  35.2,

"rain":      false,

"battery_v": 3.86,

"rssi":      -57,

"fw_ver":    "0.4.1"

}

### 9.2.2 Response (200)

{

"action":         "IRRIGATE",

"duration_sec":   15,

"next_check_min": 120,

"reason":         "Dry soil, no rain expected",

"mode":           "ONLINE",

"decision_id":    "9f2c..."

}

### 9.2.3 Response (Fallback / Schema Failure)

{

"action":         "SKIP",

"duration_sec":   0,

"next_check_min": 15,

"reason":         "LLM unavailable — safe default",

"mode":           "ONLINE-DEGRADED",

"decision_id":    "9f2c..."

}

## 9.3 ESP32 Authentication

The ESP32 uses a per-device pre-shared token (32 bytes, hex-encoded) stored in NVS at provisioning time. The token is sent in an Authorization: Device <token> header. JWT is reserved for the dashboard user, where token rotation and refresh are needed.

## 9.4 WebSocket Event Stream

The dashboard subscribes to /ws/live with the JWT in the Sec-WebSocket-Protocol header. Two event types are published: sensor.update (every time the ESP32 posts a packet) and decision.new (every time a decision is finalised). The frontend updates its SystemContext on receipt; no polling is needed.

## 9.5 Rate Limits

- Decision endpoint — 1 req / 10 s per device (token-bucket); legitimate ESP32 traffic is far below this.

- Manual control — 1 req / 2 s per user; prevents accidental rapid toggling of the relay.

- Auth — 5 attempts / minute per IP for /login.

# 10. Use Case Realizations

The following sequence sketches show how each use case from the SRS (UC-01 to UC-05) is realised against the new architecture.

## 10.1 UC-01 — Automatic Irrigation (online)

ESP32        Backend         Open-Meteo     Cerebras       Postgres

│              │                │              │              │

│ wake         │                │              │              │

│ read sensors │                │              │              │

│ POST /decision──────────────► │              │              │

│              │ read last_irr ──────────────► (returns)      │

│              │ GET forecast ─►                              │

│              │ ◄─ forecast    │              │              │

│              │ Pre-Rules pass │              │              │

│              │ chat.completions ────────────►              │

│              │ ◄─ JSON action │              │              │

│              │ Post-Rules + Pydantic         │              │

│              │ INSERT irrigation_log ─────────────────────► │

│ ◄─ 200 action│                │              │              │

│ relay ON 15 s│                │              │              │

│ deep sleep 120 min            │              │              │

## 10.2 UC-02 — Rain Postponement

Realised by Pre-Rules: when the digital rain sensor reads HIGH, the backend returns SKIP with a short next_check_min (15 min) and the LLM is never consulted. The dashboard shows the banner "Rain — irrigation postponed" because the last decision's reason field begins with "Rain detected".

## 10.3 UC-03 — Manual Control

User → Dashboard:         "Pump On"

Dashboard → Backend:      POST /control/manual {action:"ON",duration:10}

Backend → ESP32 (FCM):    push "manual.on" with signed nonce

ESP32:                    relay ON for 10 s, then resume normal mode

Backend → Postgres:       INSERT irrigation_log (mode=MANUAL, ...)

Manual mode temporarily overrides the AI. After the requested duration the system returns to its normal cycle.

## 10.4 UC-04 — Report Viewing

User → /reports?range=weekly

Dashboard → Backend:  GET /reports?range=weekly

Backend → Postgres:   aggregate irrigation_log over last 7 days

Backend ← Postgres:   {daily: [{day,liters,events}, ...]}

Dashboard:            renders Recharts BarChart and summary cards

## 10.5 UC-05 — System Fault Handling

Faults are detected at three layers and converge on the same outcome (safe mode + notification):

- Firmware — moisture reading out of physically possible range, or relay GPIO read-back disagrees with command.

- Backend — three consecutive decision-endpoint timeouts from a device; ESP32 declared offline.

- Database — system_events table receives a row of severity=DANGER; the dashboard shows a red banner; an FCM push and an email are dispatched.

# 11. Interface Design

## 11.1 User Interfaces

- Web Dashboard — described in Section 8.

- Mobile — same React build, responsive layout (Section 8.6); push notifications via Firebase Cloud Messaging.

- 1602 LCD on the device — 16 characters × 2 lines; line 1 = "M:35% PUMP:OFF", line 2 = "WiFi:ON  IRRIG".

## 11.2 Hardware Interfaces

| Component | ESP32 pin | Mode | Notes |
| --- | --- | --- | --- |
| Capacitive moisture sensor v1.2 | GPIO 34 (ADC1_CH6) | Analog | 0–3.3 V; software-calibrated to 0–100 % |
| Rain sensor (digital) | GPIO 35 | Digital | HIGH = dry, LOW = wet (open-collector); internal pull-up enabled |
| Relay module (pump) | GPIO 26 | Digital | HIGH = pump ON; opto-isolated; flyback diode on board |
| 1602 LCD (I2C) | GPIO 21 / 22 | I2C | SDA / SCL with 4.7 kΩ pull-ups; I2C address 0x27 |
| Battery voltage monitor | GPIO 32 (ADC1_CH4) | Analog | Via 100 k / 100 k voltage divider |

## 11.3 Software Interfaces

- Cerebras Inference API — POST /v1/chat/completions with Bearer token; OpenAI-compatible.

- Open-Meteo Forecast API — GET /v1/forecast, JSON, no auth.

- Firebase Cloud Messaging — server-to-device push notifications.

- SMTP relay (e.g. SendGrid free tier) — email alerts for critical events.

## 11.4 Communication Interfaces

- TLS 1.3 on all HTTP traffic. Self-signed certificates not acceptable in production; Let's Encrypt is used.

- JWT in Authorization: Bearer header for dashboard ↔ backend. Lifetime 15 min, refreshed silently.

- Device token in Authorization: Device header for ESP32 ↔ backend.

- WebSocket — wss://, JWT-authenticated, JSON envelope { type, payload, ts }.

PART II

HARDWARE DESIGN DESCRIPTION (HDD)

# 12. Hardware Architecture

## 12.1 Overview

The hardware is a single ESP32-based node powered entirely by a 6 V solar panel through a TP4056 charge controller into a single 18650 lithium-ion cell. An MT3608 step-up converter feeds 5 V to the ESP32, the relay, the LCD, and the pump. All electronics reside in a single weather-resistant enclosure (IP54 minimum) located near the plant.

## 12.2 Bill of Materials

| # | Component | Category | Qty |
| --- | --- | --- | --- |
| 1 | ESP32 Development Board | Controller | 1 |
| 2 | 6 V Mini Solar Panel | Power | 1 |
| 3 | 18650 Lithium-Ion Battery | Power | 1 |
| 4 | 18650 Single Battery Holder | Power | 1 |
| 5 | TP4056 Charging Module (Protected) | Power | 1 |
| 6 | MT3608 5 V Step-Up Module | Power | 1 |
| 7 | Capacitive Soil Moisture Sensor v1.2 | Sensor | 1 |
| 8 | Rain Sensor Module | Sensor | 1 |
| 9 | 1602 LCD Display (I2C backpack) | Display | 1 |
| 10 | 5 V Single Relay Module (Optocoupler) | Actuator | 1 |
| 11 | 5 V DC Mini Submersible Water Pump (≥120 L/h) | Actuator | 1 |
| 12 | Transparent Water Hose (0.5 m) | Mechanical | 1 |
| 13 | Full-Size Breadboard (830 holes) | Prototype | 1 |
| 14 | Perfboard (10 × 10 cm) | Prototype | 1 |
| 15 | PCB Terminal Block (2/3-pin, 5 mm pitch) | Prototype | 2 |
| 16 | Jumper Wire Set (mixed) | Prototype | 1 |

## 12.3 Hardware Functional Requirements (consolidated from PDR)

- HR-01 — ESP32 with integrated Wi-Fi + Bluetooth, dual core, ≥ 520 KB SRAM.

- HR-02 — Capacitive soil moisture sensor v1.2 (corrosion-resistant), 3.3–5 V, analog out.

- HR-03 — Rain sensor module with digital out for binary detection.

- HR-04 — 1602 character LCD with I2C backpack.

- HR-05 — 5 V single-channel relay with opto-isolation.

- HR-06 — 5 V DC submersible mini pump (≥ 120 L/h).

- HR-07 — 6 V solar panel sized to recharge the 18650 within one full sun day.

- HR-08 — 18650 Li-ion cell, ≥ 2000 mAh.

- HR-09 — TP4056 with overcharge / over-discharge / short-circuit protection.

- HR-10 — MT3608 boost converter, set to 5.00 V ±2 %.

- HR-11/12/13 — Prototyping board, jumpers, transparent hose.

- HR-14 — Enclosure IP54 minimum.

# 13. ESP32 Pin Assignment and Sensor Wiring

## 13.1 Pin Map

| Component | ESP32 pin | Interface | Notes |
| --- | --- | --- | --- |
| Capacitive moisture sensor | GPIO 34 | ADC1_CH6 | 0–3.3 V; not 5 V tolerant |
| Rain sensor (digital) | GPIO 35 | GPIO in | Internal pull-up; HIGH = dry |
| Relay module (pump) | GPIO 26 | GPIO out | HIGH = pump ON; default LOW after boot |
| 1602 LCD — SDA | GPIO 21 | I2C | Default I2C data line; 4.7 kΩ pull-up |
| 1602 LCD — SCL | GPIO 22 | I2C | Default I2C clock line; 4.7 kΩ pull-up |
| Battery voltage monitor | GPIO 32 | ADC1_CH4 | Through 100 k/100 k voltage divider |
| Status LED (optional) | GPIO 2 | GPIO out | On-board LED on most ESP32 dev kits |

## 13.2 Sensor Wiring Notes

- Moisture sensor — analog signal is read with the ADC oversampled 16× and median-filtered to suppress 50 Hz mains pickup.

- Rain sensor — internal pull-up is enabled; the open-collector output pulls LOW when wet. Software debounces with a 200 ms window.

- LCD — I2C bus is shared with no other device in this design; clock is 100 kHz; address is fixed 0x27 (PCF8574 backpack).

- Battery — the divider scales 4.20 V max to 2.10 V at the ADC, well inside the 3.3 V envelope; sampled once per cycle, not continuously.

## 13.3 Relay and Pump Circuit

- Relay module is opto-isolated; the ESP32 GPIO drives an LED on the input side, decoupling the high-current pump line.

- Pump is wired through the relay's normally-open (NO) contact and is powered directly from the 5 V rail (after MT3608).

- A 1N4007 flyback diode is present on the relay module (across the coil) for back-EMF protection.

- Maximum pump on-time is enforced both by GPIO timer in firmware and by the Post-Rules in the backend (Section 4.2).

# 14. Power System Design

## 14.1 Power Architecture

6 V Solar Panel

│

▼

┌───────────────┐

│   TP4056      │   Charge controller (overcharge / over-discharge / short)

└─────┬─────────┘

│

▼

18650 Li-ion (3.7 V nominal, 2000+ mAh)

│

▼

┌───────────────┐

│  MT3608 boost │   Output trimmed to 5.00 V

└─────┬─────────┘

│

├──► ESP32 VIN (3.3 V LDO inside)

├──► 1602 LCD (5 V)

├──► Relay coil (5 V)

└──► Pump (5 V via relay NO contact)

## 14.2 Power Budget

| Component | Active current | Sleep current | Voltage |
| --- | --- | --- | --- |
| ESP32 (Wi-Fi active) | ~ 160 mA | ~ 10 µA | 3.3 V (via 5 V VIN) |
| Capacitive moisture sensor | ~ 5 mA | — | 3.3–5 V |
| Rain sensor module | ~ 15 mA | — | 3.3–5 V |
| 1602 LCD (I2C, backlight on) | ~ 25 mA | — | 5 V |
| Relay coil (active) | ~ 70 mA | — | 5 V |
| 5 V DC mini submersible pump | ~ 130–200 mA | — | 5 V via relay |
| TP4056 charge controller | ~ 1 mA | — | 5 V solar input |
| MT3608 boost (quiescent) | ~ 1–2 mA | — | 3.7 V → 5 V |

Peak active current with all components on and the pump running is ≈ 400–480 mA. Deep sleep with only the ESP32 RTC alive is ≈ 10–50 µA, which is the dominant determinant of overnight runtime.

## 14.3 Energy Management Strategy

- Between checks the ESP32 enters deep sleep; only the RTC stays alive (~ 10 µA).

- In online mode the next-check duration is set by the LLM (10–240 min); in offline mode it is fixed at 30 min.

- Pump on-time is hard-capped to 30 s per cycle (firmware + backend).

- Battery voltage is sampled once per cycle. Below 3.3 V the system enters emergency mode (no irrigation, status reporting only). Below 3.0 V the system enters full shutdown to protect the cell.

- Deep-sleep wakeup is timer-based; no GPIO interrupts are used (the rain sensor is polled on wake).

# 15. Embedded Firmware Design (ESP32)

## 15.1 Toolchain

- PlatformIO with the espressif32 platform and the Arduino framework.

- Build configuration partitions: 1 MB OTA, 1 MB OTA, 1 MB SPIFFS, 1 MB factory.

- NVS used to store: device token, Wi-Fi credentials, plant name, calibration constants.

## 15.2 Module Decomposition

| Module | Responsibility |
| --- | --- |
| main.cpp | setup() and loop(); top-level state machine. |
| sensors/ | moistureRead(), rainRead(), batteryRead() with calibration and filtering. |
| relay.cpp | Pump on/off with hard-cap timer. |
| network/ | Wi-Fi association with timeout; HTTPS POST to backend; JSON parse via ArduinoJson. |
| fuzzy/ | Pure-C++ Fuzzy controller (membership + rule engine + centroid defuzz). |
| lcd.cpp | I2C 1602 driver thin wrapper. |
| power.cpp | esp_deep_sleep_start() with timer, low-battery checks. |

## 15.3 Top-Level State Machine

BOOT

└─ NVS read → Wi-Fi attempt (5 s timeout)

├─ success → STATE = ONLINE

└─ fail    → STATE = OFFLINE

STATE = ONLINE

└─ read sensors → POST /api/v1/decision (4 s timeout)

├─ 200      → execute action → sleep next_check_min

├─ 5xx/timeout → STATE = OFFLINE (retry online next wake)

└─ schema bad → STATE = OFFLINE

STATE = OFFLINE

└─ read sensors → run Fuzzy → execute action → sleep 30 min

└─ on next wake try Wi-Fi again

## 15.4 OTA Updates

OTA is enabled but gated. The dashboard exposes an "Update firmware" button that publishes a signed manifest URL; the ESP32 verifies the signature against a public key stored in NVS at provisioning before flashing. Without this, an attacker who learns the device token could brick the device.

## 15.5 Watchdog and Crash Recovery

- Hardware watchdog set to 30 s; resets the chip if any cycle exceeds this.

- Every reset reason is logged on next online cycle for forensics.

- On three consecutive crashes the firmware skips Wi-Fi and goes directly into offline mode for the next 6 hours, on the assumption that something in the network stack is corrupt.

PART III

QUALITY AND PROJECT MANAGEMENT

# 16. Risk Analysis and Mitigation

The PDR risk register is preserved and extended below with the failure modes uncovered during the redesign of the decision engine. Every risk has a concrete mitigation that is owned by a specific team member and is testable.

## 16.1 Risk Register

| ID | Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| R-01 | Wi-Fi loss | High | Medium | Offline Fuzzy mode (Section 5) auto resync on reconnect. |
| R-02 | Insufficient solar harvest | Medium | High | Deep sleep, low battery emergency mode, energy prioritisation. |
| R-03 | Sensor malfunction (stuck high/low) | Medium | High | 48 h timeout override (Section 16.2.1), anomaly filter, user alert. |
| R-04 | Open-Meteo unavailable | Low | Medium | Conservative mode flag in prompt, cached last known forecast. |
| R-05 | Pump or relay failure | Low | High | Hard 30 s on-time cap, runtime monitor, alert. |
| R-06 | Database performance degradation | Low | Medium | Indexes on (device_id, ts DESC), archival, statement timeout 2 s. |
| R-07 | Database write failure on hot path | Low | High | Async write + in memory queue, the action is returned regardless (Section 16.2.2). |
| R-08 | LLM JSON schema violation / hallucination | Medium | High | C# strongly typed deserialization (System.Text.Json) and FluentValidation + safe-default fallback(Section 16.2.3). |
| R-09 | Cold-boot without weather and DB | Low | Medium | Conservative lifeline irrigation, shorter next_check_min (Section 16.2.4). |
| R-10 | Cerebras free-tier revoked or rate-limited | Low | Medium | A provider independent OpenAI client (can switch between Groq and Gemini), offline mode is always available. |
| R-11 | Security breach (token leak) | Low | High | TLS, JWT, device tokens, hashed passwords, rate limited login. |
| R-12 | Project schedule overrun | Medium | High | Weekly sprints, MVP focus, the Fuzzy mode alone constitutes a working demo. |

## 16.2 Detailed Failure Modes - New Findings

### 16.2.1 R-03 - Sensor Poisoning / Hardware Stuck Output

The pre-rule “if the rain sensor is active, skip irrigation” assumes that the rain sensor is working correctly. But if the sensor is wrong for example, short circuited and always reporting rain or a corroded soil moisture sensor constantly showing 95%, the system will stay stuck in the pre rule layer. In this situation, the LLM  and the fuzzy controller never receive real sensor data, and the plant may slowly die due to lack of water.

If irrigation has been blocked by the rain prerule for 48 hours continuously, the system assumes the rain sensor is defective. The pre rule is then bypassed, a DANGER notification is triggered and the LLM (or fuzzy controller) is called with a warning flag about the sensor. The user is also asked to check the device.

### 16.2.2 R-07 - Decision Coupled to Database Write

A simple implementation might try to save the irrigation log to the database before sending the action to the ESP32. If PostgreSQL is down or the connection times out, the ESP32 would receive a 500 Internal Server Error and the plant would not be watered even though the correct decision was already made.

### Mitigation: The database write operation inside the API controller is wrapped in a try/catch block and treated as best effort. If writing fails due to a database connection issue, the error is caught and logged to the console, but the success response and action are still returned to the ESP32. Logging is important for analysis, but the main goal is to keep the plant alive.

### 16.2.3 R-08 - LLM Hallucination of JSON Keys or Types

Cerebras response_format={"type": "json_object"} ensures that the output is valid JSON but it does not guarantee that the keys or value types are correct. For example, the LLM might return something like {"action": "WATERING"} or {"duration_sec": "fifteen"}

The LLM response is deserialized into a strongly typed model. The expected format is:

- action ∈ {IRRIGATE, SKIP}

- duration_sec ∈ integer [0, 30]

- next_check_min ∈ integer [10, 240]

After deserialization, the data is validated strictly to ensure both structure and value ranges are correct. If the response does not meet these requirements, it is considered invalid.

In that case, the system falls back to a safe default decision (usually SKIP and retry in 30 minutes). It also logs the deserialization exception via ILogger and increments an OpenTelemetry error metric to track how often invalid LLM outputs occur allowing us to tweak the prompt without halting the physical hardware.

### 16.2.4 R-09 - Cold Boot Without Weather or Database

If both Open Meteo and PostgreSQL are unavailable at startup, the LLM does not have access to weather forecasts or previous irrigation data. In this case, a simple LLM might decide to water dry soil but unexpected rain shortly after could harm the plant.

The system prompt includes a flag saying Weather forecast unavailable. The LLM is instructed to act conservatively only providing minimal lifeline watering if necessary and setting a short next_check_min so the system can quickly re-evaluate when data becomes available.
If the database is also unavailable, the system falls back to the offline fuzzy controller which relies only on local sensor data.

## 16.3 Reliability Estimate

- Technical reliability: 85–90 % across all three layers, dominated by the offline mode's simplicity and the LLM's low p95 latency.

- Decision quality: 70–85 % of irrigation decisions judged reasonable against a manual reference oracle (course defined sample of 100 scenarios).

# 17. Testing Strategy

## 17.1 Test Levels

- Unit tests — On the C# side, xUnit (or NUnit) is used to test services/decision (Pre-Rules, Post-Rules), services/weather, and repos/*. The fuzzy logic module is tested in C++ using the Unity test framework, executed on the host machine (no ESP32 hardware required).

- Integration tests — backend ↔ Cerebras (using recorded HTTP responses or mock handlers to avoid unnecessary API usage); backend ↔ Postgres (Docker testcontainer); ESP32 ↔ backend on a benchtop with simulated sensor inputs.

- System tests — end-to-end scenarios: dry soil with clear forecast → IRRIGATE, dry soil with rain in 30 min → SKIP, rain sensor stuck for 48 h → override fires.

- Field tests — minimum one week of unattended outdoor operation at the project site, with photo/video evidence and a spreadsheet of decisions logged by the backend.

## 17.2 Test Matrix — Decision Engine

| Scenario | Expected action | Expected next_check |
| --- | --- | --- |
| Soil 25 %, rain in 60 min, evening | SKIP | ~ 60 min |
| Soil 25 %, clear sky, evening | IRRIGATE | ~ 90 min |
| Soil 75 %, clear sky, midday | SKIP (Pre-Rule) | short |
| Rain sensor active | SKIP (Pre-Rule) | short |
| Cerebras 502 | SKIP (degraded) | 15 min |
| Cerebras returns malformed action key | SKIP (degraded) | 15 min |
| Open-Meteo 500 | Conservative LLM | short |
| Battery 3.05 V | SKIP (emergency) | long |

## 17.3 CI

- GitHub Actions — backend tests on every push, frontend lint+build on every push, ESP32 firmware compile on every PR.

- Branch protection on main; at least one review and all checks green required to merge.

# 18. Deployment and Maintenance

## 18.1 Deployment Topology

- Backend — single VPS (1 vCPU, 1 GB RAM) behind nginx functioning as a reverse proxy with TLS termination via Let's Encrypt; Kestrel web server managed by a systemd service unit to ensure restart-on-crash functionality.

- Database — managed PostgreSQL (Supabase / Neon / Railway free tier).

- Frontend — static build deployed to Cloudflare Pages or Netlify (free tier); points at the backend domain.

- ESP32 — flashed at provisioning; receives OTA updates from the backend on user demand (Section 15.4).

## 18.2 Configuration

All environment-specific values live in a single .env on the backend host (CEREBRAS_API_KEY, DATABASE_URL, JWT_SECRET, FCM_SERVER_KEY, SMTP_*). The frontend reads only the backend URL from a build-time variable. No secrets ship in the React bundle.

## 18.3 Maintainability Guidelines

- Each layer (firmware, backend, frontend, database) is independently deployable; no in-place schema changes without a backwards-compatible window.

- Sensor calibration constants are stored in NVS on the device and editable from the dashboard, so re-flashing is not required for in-field tuning.

- A weekly cron job on the backend produces a CSV digest of decisions and emails the project owner.

# 19. Project Timeline

SRS / PDR delivery happened in week 7. The remaining six weeks focus on implementation, integration, field testing, and demo preparation.

| Week | Task | Responsible | Status |
| --- | --- | --- | --- |
| 4–5 | Requirements analysis, SRS preparation | All teams | Done |
| 6 | SRS + HRD report writing (Preliminary Design) | All teams | Done |
| 7 | PDR submission, circuit design start | All teams | Done |
| 8 | Decision-engine redesign, breadboard prototype, DB schema, API spec | Backend + Hardware | Done |
| 8 | UI/UX design (Figma) and React skeleton | Mobile/Web | Done |
| 9 | ESP32 firmware (sensors + relay + sleep), backend skeleton | Hardware + Backend | In progress |
| 9 | Web dashboard with mock data (live cards, reports) | Mobile/Web | In progress |
| 10 | Cerebras + Open-Meteo integration; offline Fuzzy in C++ | Backend + Hardware | Planned |
| 10 | Notification system (FCM push + email) | Backend + Mobile/Web | Planned |
| 11 | Power management, end-to-end integration testing | All teams | Planned |
| 12 | Field tests, bug fixes, Technical Design Report (THIS DOCUMENT) | All teams | Planned |
| 13 | Project presentations | All teams | Planned |
| 14 | Project exhibition (working demo) | All teams | Planned |

# 20. Team Structure and Responsibilities

The team consists of six members. Roles are inherited from the PDR; the responsibilities have been refined to match the redesigned architecture.

| # | Member | Role | Sections owned (TDR) |
| --- | --- | --- | --- |
| 1 | Zeynep Yamaç | Hardware/IoT — Embedded Core | 12, 13, 14, 15 |
| 2 | Berke Kocakavak | Hardware/IoT — Network | 3.4, 9, 11.4 |
| 3 | Mehmet Köksal (PM) | Backend — AI Decision | 4, 5, 6, 16 |
| 4 | Salih Akoğlu | Backend — Database | 6.3, 7, 16.2.2 |
| 5 | Şevval Özik | Mobile/Web — UI/UX | 8, 10, 11.1 |
| 6 | Ediz Gün | Mobile/Web — Frontend Integration | 8.4, 8.5, 9.4, 17.3 |

— End of Technical Design Report —